# Supplementary Table S1 — Reference cut-points for rule-based morphological categorization and Standardized Caliber Uniformity Score

Cut-points for the Complexity Score and the Standardized Caliber Uniformity Score (NV Diameter CV and dilated vessel fraction) are shown by imaging platform and field of view. Thresholds are stratum-specific and intended for within-platform classification and reporting; they do not imply biological equivalence across devices. For Optovue Solix (6×6 mm), the Complexity Score percentile reference was derived from n = 34 eyes; the primary analysis cohort in Tables 2–3 included n = 33 eyes in this stratum.

## A. Complexity Score percentile thresholds

The P30 threshold is obtained by linear interpolation between P25 and P40 and is not tabulated as a stored percentile cut-point, consistent with the morphological classification rule.

| Stratum (device) | n | P10 (Dead tree <) | P30 (Glomerular ≥; interpolated) | P40 (Seafan ≥ with SEAFAN trunk) | P65 (Medusa ≥ with MEDUSA trunk) |
| --- | --- | --- | --- | --- | --- |
| Zeiss PlexElite (6×6 mm) | 49 | 24.29 | 38.36 | 43.21 | 55.00 |
| Zeiss CIRRUS HD-OCT with AngioPlex (3×3 mm) | 30 | 44.77 | 67.93 | 69.36 | 72.85 |
| Optovue Solix (6×6 mm) | 34 | 34.20 | 41.21 | 44.93 | 53.77 |

Operational priority order: Dead tree → Medusa → Seafan → Glomerular → Tree in bud (remainder). Trunk pattern (MEDUSA / SEAFAN / INTERMEDIATE) is derived from spatial organization of large-caliber segments.

## B. Complexity Score percentiles

| Stratum | P10 | P25 | P40 | P50 | P60 | P65 | P75 | P90 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Zeiss PlexElite (6×6 mm) | 24.29 | 35.93 | 43.21 | 48.81 | 53.32 | 55.00 | 57.57 | 71.61 |
| Zeiss CIRRUS HD-OCT with AngioPlex (3×3 mm) | 44.77 | 67.21 | 69.36 | 71.03 | 72.29 | 72.85 | 74.31 | 77.74 |
| Optovue Solix (6×6 mm) | 34.20 | 39.34 | 44.93 | 48.71 | 51.06 | 53.77 | 55.65 | 65.22 |

## C. Standardized Caliber Uniformity Score

NV Diameter CV and Dilated vessel fraction cuts

Score = clip(0.75·U(−NV Diameter CV) + 0.25·U(−Dilated vessel %), 0, 100), where U maps stratum min/median/max of the negated feature to 0/50/100.

| Stratum (device) | n | NV-CV min | NV-CV median | NV-CV max | Dilated% min | Dilated% median | Dilated% max |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Zeiss PlexElite (6×6 mm) | 49 | 36.5120 | 38.4867 | 40.9939 | 0.076141 | 0.130156 | 0.183429 |
| Zeiss CIRRUS HD-OCT with AngioPlex (3×3 mm) | 30 | 43.0772 | 47.2952 | 79.1192 | 0.046519 | 0.091894 | 0.121717 |
| Optovue Solix (6×6 mm) | 33 | 37.0858 | 41.3097 | 48.8622 | 0.041908 | 0.076640 | 0.163276 |